RoboDev

ESP32-S3 Integrated Robotics Prototyping Board

Design Specification  |  v0.2.1  |  March 2026

# Revision History

| Version | Date | Summary of Changes |
| --- | --- | --- |
| v0.1 | Mar 2026 | Initial document from notebook notes (22 Oct – 17 Nov 2025) |
| v0.2 | Mar 2026 | DRV8245-Q1 → DRV8874. MPU-6050 → BMI270. |
| v0.3 | Mar 2026 | Soldered WROOM module → ESP32-S3 DevKitC-1 v1.1 in dual-row headers. |
| v0.4 | Mar 2026 | NEMA 17 stepper support: 2× TMC2209, GPIO26–31 allocated. |
| v0.5 | Mar 2026 | MPU-6050 GY-521 replaces BMI270. ADS1115, PCF8574, OLED header. Servos 8→16. SPI/UART/carrier USB-C removed. 5× ADC1 headers. |
| v0.6 | Mar 2026 | CAN bus (TWAI, GPIO7/32, SN65HVD230). Encoders 4→2. nFAULT→indicator LED. UART1 header (GPIO47/21). GPIO38/48 written off. Separate Vmot-DC and Vstep rails. 0 spare GPIOs. BOM ~NZ$82.81. |
| v0.7 | Mar 2026 | ★ I²C expansion array: 2× TCA9548A mux (0x70/0x71) + 6× Qwiic JST SH + 10× 2.54mm 4-pin headers = 16 independent expansion ports. Reverse-ARP HAL for automatic peripheral discovery. Existing 2× Qwiic absorbed. BOM ~NZ$90–92. |

# 1. Introduction

RoboDev is an open-source integrated robotics prototyping board built around the Espressif ESP32-S3 microcontroller. It is designed to eliminate the wiring overhead that typically accompanies ESP32-based robotics projects by integrating motor drivers, a servo PWM controller, stepper motor drivers, a CAN bus transceiver, an IMU, analogue ADC expanders, digital GPIO expansion, an I²C expansion array, and a complete multi-rail power supply onto a single carrier PCB.

The core philosophy of RoboDev is that only three types of wire should ever need to be connected to the board: power in, and wires out to sensors and actuators. All signal routing — from the MCU to driver ICs, from the I²C bus to peripherals, from the power switch to every rail — is handled on the PCB itself.

## 1.1 Motivation

Robotics prototyping with the ESP32 typically requires a breadboard or custom wiring harness to connect motor driver breakout boards, servo controllers, and sensor modules. This introduces noise susceptibility, mechanical fragility, and significant bring-up time before any firmware can be tested. RoboDev solves this by providing a production-ready carrier board that accepts a standard ESP32-S3 DevKitC-1 development module in removable dual-row female headers, making the MCU itself swappable without reflow.

The board targets wheeled robotic platforms, robot arms, and general mechatronics applications that require simultaneous control of multiple motor types — brushed DC motors, hobby servos, and NEMA 17 stepper motors — alongside CAN bus communication, I²C sensor feedback, and wireless connectivity.

## 1.2 Design Philosophy

- Full-featured and self-contained — no daughterboards or breakout modules required beyond the ESP32 DevKit and the MPU-6050 GY-521 module

- Any motor type supported simultaneously: 4× brushed DC, 16× servo, 2× NEMA 17 stepper, and CAN-capable motor controllers

- Maximum analogue capability — ADC1 only throughout, Wi-Fi safe on all 5 native analogue headers

- Three independently adjustable power rails for motors, steppers, and servos, allowing different voltage requirements to be met on the same board simultaneously

- ★ v0.7: 16-port I²C expansion array via 2× TCA9548A mux — 6× Qwiic (JST SH) + 10× 2.54mm 4-pin headers, all with independent buses and automatic peripheral discovery via reverse-ARP HAL

- All components hand-solderable using hot air, solder paste, stencil, or hot plate — no BGA or ultra-fine-pitch packages

- ESP32-S3 DevKitC-1 v1.1 in dual-row headers — the MCU is swappable without desoldering

- Open source — schematics, PCB layout, firmware, and documentation published on GitHub

## 1.3 Target Applications

- 2WD and 4WD wheeled robot platforms

- Multi-axis robot arms with CAN-capable joint controllers

- Mechatronics rigs requiring mixed motor types on a single controller

- Autonomous vehicles with GPS (via UART1) and IMU sensor fusion

- Sensor-dense robots using the I²C expansion array for multiple IMUs, ToF sensors, or other I²C peripherals

- Educational robotics platforms requiring a robust, well-documented base board

# 2. System Overview

The carrier PCB accepts an ESP32-S3 DevKitC-1 v1.1 in dual-row 2.54 mm female headers. The ESP32-S3's 45 available GPIOs are fully allocated across motor control, communication buses, analogue inputs, encoder feedback, and digital sensor headers. Zero spare GPIOs remain at v0.7.

v0.7 adds: A 16-port I²C expansion array implemented behind 2× TCA9548A I²C multiplexers. This is entirely zero-GPIO-cost — it shares the existing I²C bus on GPIO8/9. The expansion array provides 6 Qwiic-compatible (JST SH 1.0mm) and 10 standard 2.54mm 4-pin headers, each on an independently switched I²C channel. Address conflicts between ports are impossible by design.

The board is divided into six functional domains: power management, motor control (DC, stepper, servo), communication (I²C, CAN, UART), sensing (analogue, digital, IMU), expansion (Qwiic, GPIO headers), and the new I²C expansion array.

## 2.1 Core Requirements Summary

| Category | Requirement / Specification |
| --- | --- |
| DC Motors | 4 channels · DRV8874 · 4.5–37 V · 2.1 A cont. / 6 A peak · HTSSOP-16 · nFAULT indicator LED (autonomous, no MCU GPIO) |
| Servos | 16 channels · PCA9685 I²C PWM driver (0x40) · 12-bit resolution · Vservo rail (adj. 5–8.4 V) |
| Stepper Motors | 2 channels · TMC2209 · NEMA 17 · 2 A RMS / 2.8 A peak · UART-configurable · StealthChop2 |
| CAN Bus | TWAI (ISO 11898-1) · GPIO7 TX / GPIO32 RX · SN65HVD230 3.3 V transceiver · up to 1 Mbps · 2-pin screw terminal |
| Quadrature Encoders | 2 channels · 4-pin 2.54 mm headers · PCNT hardware quadrature (GPIO39/40, GPIO41/42) |
| Native Analogue ADC | 5 headers (A1–A5) · GPIO1,2,4,5,6 · ADC1 only · Wi-Fi safe · 12-bit |
| I²C Analogue ADC | 4 headers (A6–A9) · ADS1115 (0x48) · 16-bit · I²C |
| Total Analogue Inputs | 9 (5 native ADC1 + 4 ADS1115) |
| Digital Sensor Headers | 19 total: 8 direct ESP32 GPIO + 8 PCF8574 I²C expander + 3 freed SPI GPIO |
| UART1 Header | 4-pin 2.54 mm · GPIO47 TX / GPIO21 RX · via IO matrix · GPS, LiDAR, BT serial · up to 921600 baud |
| IMU | MPU-6050 GY-521 breakout · 6-axis (3-axis accel + 3-axis gyro) · I²C 0x68 · 4-pin header |
| OLED Display | SSD1306 0.96" header · I²C 0x3C · user-supplied module |
| GPIO Expander | PCF8574 (0x20) · 8-channel digital I/O · I²C |
| ★ I²C Expansion Array | 16 independent expansion ports · 2× TCA9548A mux (0x70/0x71) · 6× Qwiic JST SH + 10× 2.54mm 4-pin headers · reverse-ARP HAL · zero GPIO cost |
| Power Input | VIN ≥ 6 V · screw terminal + XT30 · reverse voltage protection (P-MOSFET) |
| Power Switching | High-current slide switch · switches all rails simultaneously |
| GPIO38 / GPIO48 | Written off — WS2812B RGB LED hardwired on DevKitC-1 · No Connect (NC) on carrier PCB |
| Spare GPIOs | 0 — fully allocated |
| Estimated BOM Cost | ~NZ$90–92 mid-range (excl. assembly labour, shipping, duties) |

# 3. Power Architecture

The power system is designed around five independent rails, each serving a distinct subsystem. Separating Vmot-DC from Vstep allows brushed DC motors and stepper motors to be run at different voltages simultaneously — a key requirement for mixed-actuator robotics applications. All rails are derived from a single protected VIN input, switched by a common high-current slide switch.

## 3.1 Power Input and Protection

Power enters the board via a 2-pin 5.08 mm screw terminal and/or XT30 connector. Input voltage must be at least 6 V to allow headroom for the buck converter that generates the 5 V logic rail. The maximum input voltage is limited by the lowest-rated regulator and bulk capacitors — do not exceed 36 V.

A P-channel MOSFET (AO3401) provides reverse voltage protection on VIN before the power switch. This is essential to protect all downstream components in the event of an incorrectly-wired battery.

ⓘ  A high-current slide switch is placed after the reverse voltage protection circuit. It switches all rails simultaneously. When designing the PCB layout, ensure the switch trace width is adequate for the total system current.

## 3.2 Power Rails

| Rail | Output | Current | Load | Key Component / Notes |
| --- | --- | --- | --- | --- |
| Vmot-DC | Adj. 4.5–24 V | 5 A+ cont. | DC Motors M1–M4 | e.g. XL4016/MP4560 · trimmer pot on FB · 470 µF bulk cap ×4 |
| Vstep | Adj. 4.5–24 V | 3 A+ cont. | Steppers S1–S2 | e.g. XL4015/MP4560 · independent rail for per-voltage tuning |
| Vservo | Adj. 5–8.4 V | 3 A+ cont. | Servos ×16 (PCA9685) | e.g. XL4015/MP2307 · trimmer pot · 3-pin servo headers |
| 5 V logic | Fixed 5 V | 3 A+ cont. | Dev board USB, peripherals | e.g. APS2357/MP2307 · feeds 3.3 V LDO |
| 3.3 V | Fixed 3.3 V | 1 A+ cont. | ESP32-S3, all ICs, sensors | e.g. AP2112K-3.3 LDO · all I²C devices including TCA9548A ×2 |

⚠  Vmot-DC and Vstep are independent adjustable buck converters. Each has its own trimmer potentiometer. Label both clearly on the PCB silkscreen to prevent accidental cross-connection.

⚠  The SN65HVD230 CAN transceiver operates at 3.3 V. Do not substitute a 5 V CAN transceiver such as the MCP2551 — 5 V logic will damage ESP32-S3 GPIO pins.

## 3.3 Bulk Decoupling Capacitors

Each DRV8874 DC motor driver requires at least one electrolytic bulk capacitor rated for the Vmot-DC voltage placed as close as possible to the VM pin. A minimum of 470 µF at 35 V+ is specified per driver (C1–C4). Each TMC2209 stepper driver similarly requires at least 100 µF at 35 V+ bulk capacitance (C_ST1, C_ST2). All ICs additionally receive 100 nF ceramic decoupling capacitors.

# 4. Microcontroller — ESP32-S3 DevKitC-1 v1.1

The ESP32-S3 DevKitC-1 v1.1 is a 38-pin development module from Espressif, housing the ESP32-S3-WROOM-1-N8R8 SoC (8 MB Flash, 8 MB Octal PSRAM). It plugs into two rows of 2.54 mm female headers on the carrier PCB. The row spacing is approximately 22.86 mm centre-to-centre — verify this measurement before routing PCB headers.

⚠  The v1.1 variant must be used specifically. On the v1.0 variant, the WS2812B RGB LED is connected to GPIO48 rather than GPIO38. On v1.1, it is GPIO38. Both GPIOs are written off (marked NC) on the carrier PCB, but confirming the exact variant avoids confusion during firmware bringup.

## 4.1 Key Peripherals Used

- MCPWM / LEDC — motor PWM on GPIO10–17 (DC motors M1–M4)

- TWAI (CAN 2.0) — hardware CAN controller on GPIO7 (TX) / GPIO32 (RX)

- I²C — hardware I²C on GPIO8 (SDA) / GPIO9 (SCL) with 4.7 kΩ pull-ups · shared by all on-board devices and TCA9548A expansion array

- UART1 — via IO matrix on GPIO47 (TX) / GPIO21 (RX) for GPS / LiDAR

- UART0 — debug UART via DevKit USB-UART bridge (GPIO43/44)

- PCNT (Pulse Counter) — hardware quadrature decode on GPIO39/40 and GPIO41/42

- ADC1 — 12-bit ADC on GPIO1, 2, 4, 5, 6 (Wi-Fi safe)

## 4.2 GPIO Constraints

The ESP32-S3's 45 GPIOs are not all freely usable. The following constraints apply and are reflected in the GPIO allocation:

- GPIO0, GPIO3 — strapping pins (boot mode, JTAG enable). Avoid connecting to external circuitry.

- GPIO19, GPIO20 — USB D− / D+. Reserved for USB OTG. Do not connect.

- GPIO35, GPIO36, GPIO37 — internal PSRAM octal bus. Do not connect.

- GPIO38, GPIO48 — WS2812B RGB LED hardwired on DevKitC-1. Written off; NC on carrier PCB.

- ADC2 — not used anywhere on this board. ADC2 is unreliable when Wi-Fi is active; all analogue inputs use ADC1.

# 5. GPIO Allocation

All 45 ESP32-S3 GPIOs are accounted for. 32 are actively assigned to board functions; the remainder are reserved (strapping, USB, PSRAM), used for debug UART, or written off (WS2812B LED). Zero spare GPIOs remain. The v0.7 I²C expansion array adds 16 additional peripheral ports at zero GPIO cost.

| GPIO | Function | Block | Signal | Dir. | Notes |
| --- | --- | --- | --- | --- | --- |
| GPIO1 | ADC Header A1 | ADC1 Headers | Analogue In | Input | ADC1_CH0 · Wi-Fi safe · 3-pin header GND/3.3V/SIG |
| GPIO2 | ADC Header A2 | ADC1 Headers | Analogue In | Input | ADC1_CH1 · Wi-Fi safe |
| GPIO4 | ADC Header A3 | ADC1 Headers | Analogue In | Input | ADC1_CH3 · Wi-Fi safe |
| GPIO5 | ADC Header A4 | ADC1 Headers | Analogue In | Input | ADC1_CH4 · Wi-Fi safe |
| GPIO6 | ADC Header A5 | ADC1 Headers | Analogue In | Input | ADC1_CH5 · Wi-Fi safe |
| GPIO7 | CAN TX (TWAI) | CAN Bus | Digital Out | Output | TWAI TX → SN65HVD230 TXD · freed from nFAULT (v0.6) |
| GPIO8 | I²C SDA | I²C Bus | Bidirectional | Bidir. | Shared: MPU-6050, PCA9685, ADS1115, PCF8574, OLED, TCA9548A ×2 · 4.7 kΩ pull-up |
| GPIO9 | I²C SCL | I²C Bus | Clock Out | Output | 4.7 kΩ pull-up to 3.3 V |
| GPIO10 | M1 PWM/Phase | DC Motor M1 | PWM Out | Output | DRV8874 #1 IN1 |
| GPIO11 | M1 DIR/Enable | DC Motor M1 | Digital Out | Output | DRV8874 #1 IN2 |
| GPIO12 | M2 PWM/Phase | DC Motor M2 | PWM Out | Output | DRV8874 #2 IN1 |
| GPIO13 | M2 DIR/Enable | DC Motor M2 | Digital Out | Output | DRV8874 #2 IN2 |
| GPIO14 | M3 PWM/Phase | DC Motor M3 | PWM Out | Output | DRV8874 #3 IN1 |
| GPIO15 | M3 DIR/Enable | DC Motor M3 | Digital Out | Output | DRV8874 #3 IN2 |
| GPIO16 | M4 PWM/Phase | DC Motor M4 | PWM Out | Output | DRV8874 #4 IN1 |
| GPIO17 | M4 DIR/Enable | DC Motor M4 | Digital Out | Output | DRV8874 #4 IN2 |
| GPIO18 | Digital Header | Direct GPIO | Digital I/O | Bidir. | Freed from SPI (v0.5) · general purpose |
| GPIO21 | UART1 RX | UART1 Header | Digital In | Input | UART1 via IO matrix · ⚠ minor Wi-Fi interaction reported |
| GPIO26 | Stepper 1 – STEP | Stepper S1 | PWM Out | Output | TMC2209 #1 STEP |
| GPIO27 | Stepper 1 – DIR | Stepper S1 | Digital Out | Output | TMC2209 #1 DIR |
| GPIO28 | Stepper 1 – EN | Stepper S1 | Digital Out | Output | TMC2209 #1 EN · active LOW · 10 kΩ pull-up |
| GPIO29 | Stepper 2 – STEP | Stepper S2 | PWM Out | Output | TMC2209 #2 STEP |
| GPIO30 | Stepper 2 – DIR | Stepper S2 | Digital Out | Output | TMC2209 #2 DIR |
| GPIO31 | Stepper 2 – EN | Stepper S2 | Digital Out | Output | TMC2209 #2 EN · active LOW · 10 kΩ pull-up |
| GPIO32 | CAN RX (TWAI) | CAN Bus | Digital In | Input | TWAI RX ← SN65HVD230 RXD · was spare GPIO (v0.5) |
| GPIO33 | Digital Header | Direct GPIO | Digital I/O | Bidir. | Freed SPI (v0.5) · SPI2-capable via IO matrix |
| GPIO34 | Digital Header | Direct GPIO | Digital I/O | Bidir. | Freed SPI (v0.5) |
| GPIO39 | Encoder 1 Chan A | Encoder 1 | Digital In | Input | PCNT unit 0 Chan A |
| GPIO40 | Encoder 1 Chan B | Encoder 1 | Digital In | Input | PCNT unit 0 Chan B |
| GPIO41 | Encoder 2 Chan A | Encoder 2 | Digital In | Input | PCNT unit 1 Chan A |
| GPIO42 | Encoder 2 Chan B | Encoder 2 | Digital In | Input | PCNT unit 1 Chan B |
| GPIO43 | UART0 TX (debug) | UART0/Debug | Digital Out | Output | DevKit CP2102N USB-UART bridge · serial monitor |
| GPIO44 | UART0 RX (debug) | UART0/Debug | Digital In | Input | DevKit USB-UART bridge · firmware flashing |
| GPIO47 | UART1 TX | UART1 Header | Digital Out | Output | UART1 TX via IO matrix · J4 4-pin header GND/3.3V/TX/RX |
| GPIO0 | BOOT strapping | System Reserved | Strapping | — | AVOID — boot mode selection |
| GPIO3 | JTAG strapping | System Reserved | Strapping | — | AVOID — JTAG enable |
| GPIO19 | USB D− | USB OTG | USB | — | RESERVED — USB D− |
| GPIO20 | USB D+ | USB OTG | USB | — | RESERVED — USB D+ |
| GPIO35 | PSRAM internal | Flash/PSRAM | Internal | — | RESERVED — N8R8 Octal PSRAM bus |
| GPIO36 | PSRAM internal | Flash/PSRAM | Internal | — | RESERVED — N8R8 Octal PSRAM bus |
| GPIO37 | PSRAM internal | Flash/PSRAM | Internal | — | RESERVED — N8R8 Octal PSRAM bus |
| GPIO38 | NC — WS2812B LED | Written Off | N/C | — | WRITTEN OFF — RGB LED hardwired on DevKitC-1 v1.1. Do not connect on carrier PCB. |
| GPIO48 | NC — WS2812B LED | Written Off | N/C | — | WRITTEN OFF — RGB LED hardwired (v1.0=GPIO48). Do not connect on carrier PCB. |

# 6. Motor Control

## 6.1 DC Brushed Motors — DRV8874 × 4

Four DRV8874 H-bridge motor driver ICs (U2–U5) provide control for four independent brushed DC motor channels (M1–M4). The DRV8874 is a fully integrated H-bridge with hardware overcurrent protection, thermal shutdown, and undervoltage lockout.

| Parameter | Value / Detail |
| --- | --- |
| Part number | DRV8874PWPR (HTSSOP-16) |
| Supply voltage (VM) | 4.5 V – 37 V (Vmot-DC rail) |
| Continuous current | 2.1 A per channel |
| Peak current | 6 A per channel |
| Control signals M1 | GPIO10 (PWM/Phase), GPIO11 (DIR/Enable) |
| Control signals M2 | GPIO12 (PWM/Phase), GPIO13 (DIR/Enable) |
| Control signals M3 | GPIO14 (PWM/Phase), GPIO15 (DIR/Enable) |
| Control signals M4 | GPIO16 (PWM/Phase), GPIO17 (DIR/Enable) |
| Output connector | 2-pin 5.08 mm screw terminal per channel (J5–J8) |
| nFAULT handling | All 4 nFAULT pins wired-OR open-drain → red indicator LED. No MCU GPIO consumed. Protection (OCP, TSD, UVLO) is autonomous within the DRV8874. |
| Bulk capacitance | 470 µF 35 V+ electrolytic per driver (C1–C4), placed close to VM pin |

## 6.2 NEMA 17 Stepper Motors — TMC2209 × 2

Two TMC2209 silent stepper motor driver ICs (U14–U15) drive two NEMA 17 bipolar stepper motors (S1–S2) from the independent Vstep rail. The TMC2209 features StealthChop2 for near-silent operation and SpreadCycle for high-torque applications, configurable via UART.

| Parameter | Value / Detail |
| --- | --- |
| Part number | TMC2209-LA-T (QFN-28) |
| Supply voltage (VM) | 4.5 V – 36 V (Vstep rail — independent from Vmot-DC) |
| RMS current | 2.0 A RMS per channel |
| Peak current | 2.8 A peak per channel |
| Control S1 | GPIO26 (STEP), GPIO27 (DIR), GPIO28 (EN, active LOW) |
| Control S2 | GPIO29 (STEP), GPIO30 (DIR), GPIO31 (EN, active LOW) |
| EN pin pull-up | 10 kΩ to 3.3 V — drivers disabled by default at power-on |
| Output connector | 4-pin 2.54 mm STEP/DIR/EN header per channel (J54–J55) |
| Bulk capacitance | 100 µF 35 V+ per driver (C_ST1, C_ST2) |
| Configuration | UART-configurable via TMC2209 PDN_UART pin (optional firmware feature) |

ⓘ  The Vstep rail is intentionally separate from Vmot-DC. This allows, for example, 12 V NEMA 17 operation alongside 6 V DC gearmotors on the same board without compromise.

## 6.3 Servo Motors — PCA9685 × 16

A PCA9685 16-channel 12-bit PWM driver IC (U8) provides servo control for up to 16 hobby servo channels simultaneously via I²C. All 16 PWM outputs are broken out to 3-pin 2.54 mm servo headers (J28–J43) carrying GND, Vservo, and PWM signal.

| Parameter | Value / Detail |
| --- | --- |
| Part number | PCA9685PW (TSSOP-28) |
| I²C address | 0x40 (ADDR pins → GND) |
| PWM resolution | 12-bit (4096 steps) |
| PWM frequency | Configurable 24 Hz – 1526 Hz (typically 50 Hz for standard servos) |
| Servo voltage rail | Vservo (adj. 5–8.4 V, independent adjustable buck, ≥3 A) |
| Servo headers | J28–J43: 3-pin 2.54 mm GND / Vservo / PWM (16 channels) |
| Control interface | I²C via GPIO8 (SDA) / GPIO9 (SCL) |

## 6.4 Motor Compatibility

| Motor Type | Compatible? | Notes |
| --- | --- | --- |
| Pololu micro/mini DC gearmotors | ✓ Yes | Well within DRV8874 2.1 A continuous rating |
| TT yellow gearmotors (2WD/4WD) | ✓ Yes | Common chassis motors — ideal fit |
| 25 mm / 37 mm DC gearmotors | ✓ Yes* | Stall may approach 6 A peak — current limiting recommended |
| 775-size DC motors | ✗ No | Exceeds DRV8874 continuous rating — use external driver |
| Hobby servos (3–8.4 V) | ✓ Yes | PCA9685 servo rail, up to 16 channels simultaneously |
| NEMA 17 stepper motors | ✓ Yes | TMC2209 StealthChop2 · up to 2 A RMS / 2.8 A peak |
| CAN motor controllers | ✓ Yes | TWAI on GPIO7/32 · SN65HVD230 · ISO 11898-1 |

# 7. Communication Interfaces

## 7.1 I²C Bus

A single I²C bus on GPIO8 (SDA) and GPIO9 (SCL) carries all on-board I²C peripherals plus the v0.7 TCA9548A expansion array. Pull-up resistors of 4.7 kΩ to 3.3 V are provided on-board. No external pull-ups are needed for any on-board device.

| Device | I²C Addr | Function | Header / Notes | Bus Topology |
| --- | --- | --- | --- | --- |
| MPU-6050 GY-521 | 0x68 | 6-axis IMU (3-axis accel + 3-axis gyro) | 4-pin 2.54 mm header J_IMU | Main bus — always visible |
| PCA9685 | 0x40 | 16-channel 12-bit PWM servo driver | IC on PCB · 16× 3-pin servo headers | Main bus — always visible |
| ADS1115 | 0x48 | 4-channel 16-bit ADC (A6–A9) | IC on PCB · ADDR→GND | Main bus — always visible |
| PCF8574 | 0x20 | 8-channel digital GPIO expander | IC on PCB · INT not connected | Main bus — always visible |
| SSD1306 OLED | 0x3C | 0.96" OLED display (user-supplied) | 4-pin 2.54 mm header | Main bus — always visible |
| TCA9548A Mux #1 | 0x70 | I²C mux — expansion ports 0–7 | IC on PCB | ★ NEW v0.7 · Main bus — always visible |
| TCA9548A Mux #2 | 0x71 | I²C mux — expansion ports 8–15 | IC on PCB | ★ NEW v0.7 · Main bus — always visible |
| Expansion ports 0–15 | Various | User I²C devices (any address) | 4× Qwiic (JST SH) + 4× 2.54mm (Mux #1); 2× Qwiic + 6× 2.54mm (Mux #2) | ★ NEW v0.7 · Behind mux — no address conflicts between ports |

ⓘ  There are no I²C address conflicts between the seven on-board devices. The two TCA9548A muxes occupy 0x70 and 0x71. Devices connected to expansion ports cannot conflict with each other because each port is on an independently switched bus channel.

## 7.2 CAN Bus (TWAI)

CAN bus support is implemented using the ESP32-S3's built-in TWAI (Two-Wire Automotive Interface) hardware controller, conforming to ISO 11898-1 (CAN 2.0A/B). An SN65HVD230 3.3 V CAN transceiver (U6) provides the physical layer.

| Parameter | Value / Detail |
| --- | --- |
| Protocol | ISO 11898-1 (CAN 2.0A/B) via ESP32-S3 TWAI hardware controller |
| Transceiver | SN65HVD230 (3.3 V compatible, up to 1 Mbps) |
| TX pin | GPIO7 (previously nFAULT in v0.5 — freed by moving fault to LED in v0.6) |
| RX pin | GPIO32 (previously last spare GPIO in v0.5) |
| Bus connector | 2-pin 5.08 mm screw terminal (CANH / CANL) |
| Termination | 120 Ω resistor footprint on board with solder-bridge enable/disable |
| Logic level | 3.3 V — directly compatible with ESP32-S3 GPIO |
| Use cases | CAN motor controllers (VESC, ODrive), robot arm joints, CAN sensors, inter-board comms |

⚠  Do not substitute the SN65HVD230 with a 5 V transceiver such as the MCP2551. The ESP32-S3 GPIO pins are 3.3 V tolerant only; a 5 V transceiver output will permanently damage GPIO32.

## 7.3 UART1 Header

A dedicated UART1 header (J4) provides a 4-pin 2.54 mm interface for serial communication with GPS modules, LiDAR sensors, Bluetooth serial adapters, and any UART-based peripheral. UART1 is routed via the ESP32-S3 IO matrix.

| Parameter | Value / Detail |
| --- | --- |
| TX pin | GPIO47 (UART1 TX via IO matrix) |
| RX pin | GPIO21 (UART1 RX via IO matrix) |
| Header | J4: 4-pin 2.54 mm — GND / 3.3V / TX / RX |
| Baud rate | Configurable in firmware — standard rates up to 921600 baud |
| Logic level | 3.3 V — use level shifter for 5 V devices |
| Use cases | GPS (NMEA), LiDAR (RPLIDAR, TF-Luna), Bluetooth serial (HC-05/06) |

ⓘ  GPIO21 has reported minor Wi-Fi interaction on some ESP32-S3 modules. Test UART1 RX reliability with Wi-Fi active. For GPS use cases this is generally not an issue as GPS baud rates are low (9600–115200 baud).

## 7.4 Debug UART (UART0)

UART0 (GPIO43 TX / GPIO44 RX) is connected to the ESP32-S3 DevKitC-1's onboard CP2102N USB-UART bridge. This provides the standard USB-C programming and serial monitor interface. No additional connections are needed for firmware flashing or serial debugging.

# 8. Sensing and Analogue Inputs

## 8.1 Inertial Measurement Unit — MPU-6050

An MPU-6050 GY-521 breakout module (U7) provides 6-axis inertial measurement (3-axis accelerometer + 3-axis gyroscope) on a dedicated 4-pin I²C header. The GY-521 module includes an onboard 3.3 V regulator and I²C level shifters, making it directly compatible with the board's 3.3 V I²C bus.

- I²C address: 0x68 (AD0 pin low)

- Header: J_IMU — 4-pin 2.54 mm GND / 3.3V / SDA / SCL

- Firmware note: a Kalman filter or complementary filter is recommended for stable angle estimation

## 8.2 Native Analogue Inputs (ADC1) — A1 to A5

Five 3-pin 2.54 mm analogue headers (J44–J49) provide direct ADC1 inputs from the ESP32-S3. All five use ADC1 exclusively — ADC2 is not used anywhere on this board, as ADC2 is unreliable when Wi-Fi is active.

- A1: GPIO1 (ADC1_CH0)  · A2: GPIO2 (ADC1_CH1)  · A3: GPIO4 (ADC1_CH3)  · A4: GPIO5 (ADC1_CH4)  · A5: GPIO6 (ADC1_CH5)

- Pinout: GND / 3.3V / SIG (3-pin header)

- Resolution: 12-bit (0–4095)  |  Input range: 0–3.3 V  |  Wi-Fi safe

## 8.3 I²C Analogue Inputs (ADS1115) — A6 to A9

An ADS1115 16-bit I²C ADC (U9) provides four additional analogue input channels (A6–A9) at significantly higher resolution than the native ADC1. This is particularly well-suited for DRV8874 IPROPI current sense monitoring (see Section 11 — Recommended Improvements).

- I²C address: 0x48 (ADDR pin → GND)

- Resolution: 16-bit (±32767 counts)  |  Programmable gain: ±0.256 V to ±6.144 V FSR

- Headers: J50–J53 — 3-pin 2.54 mm GND / 3.3V / SIG (4 channels)

## 8.4 Digital Sensor Headers

Nineteen digital sensor headers are available in three categories:

- 8 × direct ESP32-S3 GPIO (GPIO18, 33, 34 + freed SPI pins) — 4-pin headers GND/3.3V/5V/SIG

- 8 × PCF8574 I²C GPIO expander channels — 4-pin headers GND/3.3V/5V/SIG (zero MCU GPIO cost)

- 3 × additional freed SPI GPIO — broken out to 4-pin headers

## 8.5 Quadrature Encoders

Two quadrature encoder channels are supported using the ESP32-S3's hardware PCNT (Pulse Counter) units. Each channel provides a 4-pin 2.54 mm header.

- Encoder 1: GPIO39 (Chan A), GPIO40 (Chan B) — PCNT unit 0

- Encoder 2: GPIO41 (Chan A), GPIO42 (Chan B) — PCNT unit 1

- Pinout: GND / 3.3V / Chan-A / Chan-B (4-pin header)

ⓘ  Encoder count was reduced from 4 to 2 in v0.6. The freed GPIO pairs (47/21 and 48/38) were reassigned to UART1 and written off respectively.

# 9. I²C Expansion Array  ★ New v0.7

★  NEW v0.7  This entire section is new in v0.7. The I²C expansion array adds 16 independent expansion ports at zero GPIO cost by multiplexing the existing I²C bus through two TCA9548A 8-channel mux ICs.

The v0.7 I²C expansion array is the centrepiece change in this revision. Two TCA9548A 8-channel I²C multiplexers (U_MUX1 at 0x70 and U_MUX2 at 0x71) are added to the main I²C bus, each providing 8 independently switched downstream channels. This gives a total of 16 expansion ports — each port is a fully isolated I²C segment.

Because each TCA9548A channel is switched independently, devices connected to different ports can share the same I²C address without conflict. For example, 16 SSD1306 displays (all at 0x3C) could be connected to ports 0–15 and accessed one at a time without any conflict.

## 9.1 Expansion Port Layout

| Port | Connector | Mux Channel | Bus Isolation | Address | Notes |
| --- | --- | --- | --- | --- | --- |
| 0 | Qwiic JST SH 4-pin | TCA9548A #1 Ch0 | Independent | Any | ★ Qwiic port 1 of 4 on Mux #1 |
| 1 | Qwiic JST SH 4-pin | TCA9548A #1 Ch1 | Independent | Any | ★ Qwiic port 2 of 4 on Mux #1 |
| 2 | Qwiic JST SH 4-pin | TCA9548A #1 Ch2 | Independent | Any | ★ Qwiic port 3 of 4 on Mux #1 |
| 3 | Qwiic JST SH 4-pin | TCA9548A #1 Ch3 | Independent | Any | ★ Qwiic port 4 of 4 on Mux #1 |
| 4 | 2.54mm 4-pin header | TCA9548A #1 Ch4 | Independent | Any | ★ 2.54mm port 1 of 4 on Mux #1 · GND/3.3V/SDA/SCL |
| 5 | 2.54mm 4-pin header | TCA9548A #1 Ch5 | Independent | Any | ★ 2.54mm port 2 of 4 on Mux #1 |
| 6 | 2.54mm 4-pin header | TCA9548A #1 Ch6 | Independent | Any | ★ 2.54mm port 3 of 4 on Mux #1 |
| 7 | 2.54mm 4-pin header | TCA9548A #1 Ch7 | Independent | Any | ★ 2.54mm port 4 of 4 on Mux #1 |
| 8 | Qwiic JST SH 4-pin | TCA9548A #2 Ch0 | Independent | Any | ★ Qwiic port 1 of 2 on Mux #2 |
| 9 | Qwiic JST SH 4-pin | TCA9548A #2 Ch1 | Independent | Any | ★ Qwiic port 2 of 2 on Mux #2 |
| 10 | 2.54mm 4-pin header | TCA9548A #2 Ch2 | Independent | Any | ★ 2.54mm port 1 of 6 on Mux #2 |
| 11 | 2.54mm 4-pin header | TCA9548A #2 Ch3 | Independent | Any | ★ 2.54mm port 2 of 6 on Mux #2 |
| 12 | 2.54mm 4-pin header | TCA9548A #2 Ch4 | Independent | Any | ★ 2.54mm port 3 of 6 on Mux #2 |
| 13 | 2.54mm 4-pin header | TCA9548A #2 Ch5 | Independent | Any | ★ 2.54mm port 4 of 6 on Mux #2 |
| 14 | 2.54mm 4-pin header | TCA9548A #2 Ch6 | Independent | Any | ★ 2.54mm port 5 of 6 on Mux #2 |
| 15 | 2.54mm 4-pin header | TCA9548A #2 Ch7 | Independent | Any | ★ 2.54mm port 6 of 6 on Mux #2 |

## 9.2 TCA9548A Mux Architecture

| Parameter | Value / Detail |
| --- | --- |
| Part number | TCA9548APWR (TSSOP-24) |
| I²C address Mux #1 | 0x70 (A2/A1/A0 = 0/0/0 → GND) |
| I²C address Mux #2 | 0x71 (A2/A1/A0 = 0/0/1 → Mux#2 A0 → 3.3V) |
| Channels per IC | 8 independent I²C downstream channels |
| Total expansion ports | 16 (8 from Mux #1 + 8 from Mux #2) |
| Connector types | Mux #1: 4× Qwiic (JST SH) + 4× 2.54mm 4-pin; Mux #2: 2× Qwiic + 6× 2.54mm 4-pin |
| Supply voltage | 3.3 V — from 3.3 V logic rail |
| Pull-up resistors | 4.7 kΩ on SDA/SCL of main bus only. No additional pull-ups needed on expansion ports (mux output is active). |
| Address conflict risk | None between ports — each port is on an independently switched bus segment |

## 9.3 Reverse-ARP HAL — Automatic Peripheral Discovery

The v0.7 firmware Hardware Abstraction Layer (HAL) implements a reverse-ARP (Address Resolution Protocol) discovery mechanism for the expansion array. On startup, the HAL iterates through all 16 expansion ports, temporarily enables each channel, performs an I²C scan, records any device addresses found, and then disables the channel.

The result is a port map — a data structure that records which devices are present on which port. The HAL then exposes named handles (i2c_port_0 through i2c_port_15) that application code can use to communicate with a specific port. The HAL handles all TCA9548A channel switching transparently; application code never calls the mux directly.

- On startup: scan all 16 ports → build port map → log device addresses found per port

- At runtime: application requests i2c_port_N → HAL enables TCA9548A channel → performs I²C transaction → disables channel

- No address conflicts possible between ports — HAL enforces single-channel activation

- Fixed on-board devices (MPU-6050, PCA9685, ADS1115, PCF8574, OLED) remain on the main bus and are always accessible without mux switching

⚠  Only one TCA9548A channel should be active at any time. The HAL enforces this. Do not attempt to manually control the TCA9548A channel registers from application code while using the HAL — this will confuse the port map.

# 10. Connector Reference

All connectors on the carrier PCB are summarised below, including reference designators, types, quantities, and pinouts.

| Ref | Type | Qty | Pitch | Function / Pinout |
| --- | --- | --- | --- | --- |
| J5–J8 | Screw terminal | 4 | 5.08mm 2-pin | DC motor outputs M1–M4 (+ / −) |
| J_CAN | Screw terminal | 1 | 5.08mm 2-pin | CAN bus: CANH / CANL |
| J9 | Screw terminal | 1 | 5.08mm 2-pin | VIN power input (+ / −) |
| J10–J11 | Pin header | 2 | 2.54mm 4-pin | Encoders 1 & 2: GND / 3.3V / Chan-A / Chan-B |
| J4 | Pin header | 1 | 2.54mm 4-pin | UART1: GND / 3.3V / TX(GPIO47) / RX(GPIO21) |
| J_IMU | Pin header | 1 | 2.54mm 4-pin | MPU-6050 GY-521: GND / 3.3V / SDA / SCL |
| J3 | Pin header | 1 | 2.54mm 4-pin | OLED SSD1306: GND / 3.3V / SDA / SCL |
| J54–J55 | Pin header | 2 | 2.54mm 4-pin | Stepper STEP/DIR/EN headers (×2) |
| J12–J19 | Pin header | 8 | 2.54mm 4-pin | Direct GPIO sensor headers: GND / 3.3V / 5V / SIG |
| J20–J27 | Pin header | 8 | 2.54mm 4-pin | PCF8574 digital headers: GND / 3.3V / 5V / SIG |
| J28–J43 | Pin header | 16 | 2.54mm 3-pin | Servo headers: GND / Vservo / PWM (all PCA9685 ch.) |
| J44–J49 | Pin header | 5 | 2.54mm 3-pin | ADC1 analogue A1–A5: GND / 3.3V / SIG |
| J50–J53 | Pin header | 4 | 2.54mm 3-pin | ADS1115 analogue A6–A9: GND / 3.3V / SIG |
| ★ EXP_Q1–Q6 | JST SH 4-pin | 6 | 1.0mm Qwiic | ★ NEW · I²C expansion Qwiic ports (GND/3.3V/SDA/SCL) |
| ★ EXP_H1–H10 | Pin header | 10 | 2.54mm 4-pin | ★ NEW · I²C expansion 2.54mm ports (GND/3.3V/SDA/SCL) |
| J1–J2 | JST SH 4-pin | 2 | 1.0mm Qwiic | Note: now routed through TCA9548A expansion array |
| J_MCU_A/B | Female header | 2 | 2.54mm 19-pin | ESP32-S3 DevKitC-1 dual-row socket (~22.86mm spacing) |

# 11. Bill of Materials (v0.7)

Estimated mid-range unit pricing at Qty=1 board. USD→NZD rate 1.6887 (XE.com, 11 Mar 2026). Excludes PCB assembly labour, shipping, and import duties. Green rows are new in v0.7.

| # | Component | Part No. / Variant | Qty | Unit NZD | Notes |
| --- | --- | --- | --- | --- | --- |
| 1 | ESP32-S3 DevKitC-1 v1.1 | ESP32-S3-DevKitC-1 v1.1 | 1 | ~$9.71 | ⚠ Must be v1.1 specifically |
| 2 | Female Pin Header 2.54mm 19-pin | Wurth 61301921121 | 2 | ~$0.63 | MCU socket (2 rows) |
| 3 | DRV8874 H-Bridge Motor Driver | DRV8874PWPR (HTSSOP-16) | 4 | ~$2.96 | DC motors M1–M4 |
| 4 | SN65HVD230 CAN Transceiver | SN65HVD230DR (SOIC-8) | 1 | ~$2.03 | CAN bus |
| 5 | CAN Screw Terminal 5.08mm 2-pin | Phoenix MKDS 1.5/2 | 2 | ~$0.51 | ×1 CANH/CANL, ×1 spare |
| 6 | nFAULT Red LED SMD | LTST-C190KRKT | 1 | ~$0.17 | DRV8874 fault indicator |
| 7 | MPU-6050 GY-521 Breakout Module | GY-521 | 1 | ~$2.53 | 6-axis IMU |
| 8 | PCA9685 16-ch I²C PWM Driver | PCA9685PW (TSSOP-28) | 1 | ~$2.11 | Servo driver |
| 9 | ADS1115 16-bit I²C ADC | ADS1115IDGST (MSOP-10) | 1 | ~$3.38 | Analogue headers A6–A9 |
| 10 | PCF8574 I²C GPIO Expander | PCF8574AP | 1 | ~$1.35 | Digital sensor headers |
| ★11 | TCA9548A I²C Mux (8-ch) | TCA9548APWR (TSSOP-24) | 2 | ~$2.53 | ★ NEW · Mux #1 addr 0x70, Mux #2 addr 0x71 |
| ★12 | Qwiic JST SH 1.0mm 4-pin Header | JST SH SM04B-SRSS-TB | 6 | ~$0.25 | ★ NEW · 6 Qwiic ports on expansion array |
| ★13 | Pin Header 2.54mm 4-pin (expansion) | Wurth 61300411121 | 10 | ~$0.08 | ★ NEW · 10 expansion headers GND/3.3V/SDA/SCL |
| 14 | 5V Synchronous Buck (≥3A) | APS2357 / MP2307 | 1 | ~$1.65 | Logic 5V rail |
| 15 | 3.3V LDO (≥1A) | AP2112K-3.3 (SOT-25) | 1 | ~$0.59 | Logic 3.3V |
| 16 | Vservo Adj. Buck 5–8.4V (≥3A) | XL4015 / MP4560 | 1 | ~$1.77 | Servo rail |
| 17 | Vmot-DC Adj. Buck 4.5–24V (≥5A) | XL4016 / MP4560 | 1 | ~$1.77 | DC motor rail |
| 18 | Vstep Adj. Buck 4.5–24V (≥3A) | XL4015 / MP4560 | 1 | ~$1.77 | Stepper rail |
| 19 | P-ch MOSFET Rev. Voltage Prot. | AO3401 (SOT-23) | 1 | ~$0.34 | VIN protection |
| 20 | High-Current Slide Switch | SS12F44G4 | 1 | ~$0.59 | Main power switch |
| 21 | Electrolytic Cap 470µF 35V+ | Panasonic EEU-FR1V471 | 4 | ~$0.51 | DC motor bulk cap × 4 |
| 22 | Electrolytic Cap 100µF 35V+ | Panasonic EEU-FR1V101 | 2 | ~$0.51 | Stepper bulk cap × 2 |
| 23 | TMC2209 Silent Stepper Driver | TMC2209-LA-T (QFN-28) | 2 | ~$4.22 | NEMA17 S1/S2 |
| 24 | 100nF Ceramic Decoupling (×30) | GRM188R71C104KA01D | 30 | ~$0.03 | All IC bypass caps |
| 25 | Resistor assortment 0402/0603 (×40) | Misc. | 40 | ~$0.03 | Pull-ups, LEDs, dividers |
| 26 | 10kΩ Multi-turn Trimmer Pot | Bourns 3296W-1-103LF | 1 | ~$0.76 | Servo buck FB adjust |
| 27 | PCB 2-layer ~130×95mm (5 pcs) | JLCPCB / PCBWay | 1 | ~$16.00 | Slightly larger for expansion headers |

Estimated total board cost (1× assembled, mid-range): ~NZ$90–92  (v0.6 was ~NZ$82.81 · delta: 2× TCA9548A ~$5.06, 6× Qwiic headers ~$1.50, 10× 2.54mm hdrs ~$0.80, larger PCB ~$0.80)

# 12. Recommended Improvements

The following improvements are identified for future revisions. None require additional GPIOs except where noted — the board currently has 0 spare GPIOs.

| Improvement | Priority | Est. Cost | Notes |
| --- | --- | --- | --- |
| Battery voltage monitor | High ⭐ | <NZ$0.10 | Resistor divider on VIN → any ADC1 header. Zero GPIO cost. |
| Per-motor IPROPI current sense | Medium ◆ | ~NZ$0.20 | Route DRV8874 IPROPI → ADS1115 inputs. 16-bit resolution. Excellent fit — 4 inputs, 4 motors. |
| LiPo XT30 connector | Medium ◆ | ~NZ$0.30 | Alongside VIN screw terminal. Zero GPIO cost. |
| On-board passive buzzer | Low ◇ | ~NZ$0.30 | No spare GPIOs — would require sacrificing a digital header pin. |
| WS2812B RGB status LED | Low ◇ | ~NZ$0.15 | No spare GPIOs — same constraint as buzzer. |

# 13. Open Questions and Next Steps

## 13.1 Verification Required

- Confirm ESP32-S3 DevKitC-1 v1.1 specifically — verify GPIO38 = WS2812B LED (not GPIO48 as in v1.0)

- Measure DevKitC-1 physical row spacing before routing carrier PCB headers (~22.86 mm expected)

- Prototype I²C bus on breadboard — verify all 7 device addresses and pull-up values at full bus loading (including both TCA9548A)

- Prototype CAN bus — verify SN65HVD230 wiring and TWAI firmware configuration (bitrate, mode)

- Test UART1 RX reliability (GPIO21) with Wi-Fi active in target applications

- Verify TCA9548A address selection resistors for 0x70 (all pins → GND) and 0x71 (A0 → 3.3V)

## 13.2 PCB Design Tasks

- Create KiCad or EasyEDA schematic — start with power system, then I²C bus, then motor drivers

- Add 120 Ω CAN termination resistor footprint with solder-bridge enable/disable

- Plan ADS1115 input routing for per-motor IPROPI current sense at schematic stage

- Mark GPIO38 and GPIO48 as NC (no connect) on carrier PCB silkscreen

- Ensure Vmot-DC and Vstep rails are clearly labelled on silkscreen to prevent mismatch

- Separate ground planes: high-current motor ground physically isolated from logic ground with single star-point connection

- ★ Route TCA9548A mux ICs to expansion headers — ensure SDA/SCL lines to each expansion port are clearly routed and labelled on silkscreen

- ★ Confirm PCB size increase to ~130×95mm to accommodate 16 expansion port headers

## 13.3 Firmware

- Generate LCSC / Mouser priced BOM from this document for procurement

- Implement basic bringup firmware: I²C scan (all 16 expansion ports), PWM test, CAN loopback, UART echo

- Implement Kalman/complementary filter for MPU-6050 angle estimation

- Consider UART-based TMC2209 configuration for per-application current tuning

- ★ Implement reverse-ARP HAL for TCA9548A expansion array — port map on startup, transparent channel switching at runtime

- ★ Define HAL API: i2c_port_scan_all(), i2c_port_read(port, addr, reg, len), i2c_port_write(port, addr, reg, data, len)

# Appendix A — GPIO Budget Summary

| Category | GPIO Count | Assignment |
| --- | --- | --- |
| DC Motors — PWM + DIR (4 ch) | 8 | GPIO10–17 |
| CAN Bus TX/RX (TWAI) | 2 | GPIO7 (TX), GPIO32 (RX) |
| Quadrature Encoders (2 ch) | 4 | GPIO39/40, GPIO41/42 |
| I²C Bus (SDA + SCL) | 2 | GPIO8, GPIO9 |
| Native ADC1 headers A1–A5 | 5 | GPIO1, 2, 4, 5, 6 |
| Stepper STEP/DIR/EN (2 ch) | 6 | GPIO26–31 |
| UART1 header | 2 | GPIO47 (TX), GPIO21 (RX) |
| Direct digital GPIO headers | 3 | GPIO18, 33, 34 |
| TOTAL ESP32 GPIO ASSIGNED | 32 | — |
| Reserved (strapping/USB/PSRAM) | 8+ | GPIO0, 3, 19, 20, 35, 36, 37 |
| UART0 debug (DevKit USB bridge) | 2 | GPIO43, 44 |
| Written off (WS2812B LED) | 2 | GPIO38, GPIO48 — NC on carrier PCB |
| SPARE | 0 | No spare GPIOs — fully allocated |
| — I²C PERIPHERALS (zero GPIO cost) — |  |  |
| I²C devices on main bus | 7 | MPU-6050, PCA9685, ADS1115, PCF8574, SSD1306, TCA9548A ×2 |
| ADS1115 analogue inputs (A6–A9) | 4 | 16-bit resolution — zero ESP32 GPIO cost |
| PCF8574 digital I/O pins | 8 | Digital I/O via I²C — zero GPIO cost |
| Total servo channels (PCA9685) | 16 | All PCA9685 channels — zero GPIO cost |
| ★ Expansion ports (TCA9548A ×2) | 16 | 6× Qwiic + 10× 2.54mm — zero GPIO cost |
| Total analogue inputs | 9 | 5× native ADC1 + 4× ADS1115 |

# Appendix B — I²C Address Map

| Address | Device | Bus Location | Notes |
| --- | --- | --- | --- |
| 0x20 | PCF8574 | Main bus (fixed) | 8-channel digital GPIO expander |
| 0x3C | SSD1306 OLED | Main bus (fixed) | 0.96" OLED header — user-supplied |
| 0x40 | PCA9685 | Main bus (fixed) | 16-ch servo PWM driver |
| 0x48 | ADS1115 | Main bus (fixed) | 4× 16-bit analogue ADC |
| 0x68 | MPU-6050 | Main bus (fixed) | 6-axis IMU |
| 0x70 | TCA9548A Mux #1 | Main bus (fixed) | ★ NEW · 8-channel I²C mux · ports 0–7 |
| 0x71 | TCA9548A Mux #2 | Main bus (fixed) | ★ NEW · 8-channel I²C mux · ports 8–15 |
| Various | User devices | Expansion ports 0–15 | ★ NEW · No conflict between ports — each port isolated |